To complete the clause, select the appropriate wording from below. Highlighted text can be ‘dragged and dropped’ to save typing.

N10 GENERAL FIXTURES/ FURNISHINGS/ EQUIPMENT

300 ENTRANCE MATTING:

Nuway Connect is a single sided entrance system ideal for entrance areas subject to high levels of pedestrian traffic and where soiling has a high moisture or dust content. Available in open and closed constructions, Connect can be shaped on site and offers a choice of textile, 4 row bristle, rubber and Ultragrip safety rubber inserts, all of which can be replaced on site helping to extend the wear life of the product.

Nuway Connect is available in 10mm, 17mm and 22mm thickness.

A choice of 6 textile infills is available for interior use with fluted rubber or Ultragrip safety rubber insets and 4 row bristles available for interior and exterior use.  Black Ultragrip rubber is suitable for use in areas where the matting will be subject to multi directional traffic. Fluted rubber is advised for use where mats are primarily subject to single direction traffic (at right angles to the direction of the ribs).

Ultragrip rubber inserts have been tested using the Pendulum (the HSE recommended test method) in accordance with BS7976-2 and the UK Slip Resistance Group Guidelines and offer a low slip risk in wet and dry conditions.

All inserts can be mixed and matched to create the exact system to meet the specification

In open construction, the mat is rollable for ease of cleaning of the matwell.

Connect with Coral inserts meets Bfl-S1

Consult with Forbo technical literature for further details.

300 ENTRANCE MATTING:

Manufacturer: Forbo Flooring UK Ltd, P O Box 1, Kirkcaldy Fife, KY1 2SB.

Tel: 01773 740 688 Fax: 01773 740 640

Email: info.flooring.uk@forbo.com. Web: www.forbo-flooring.co.uk

Product reference: Nuway Connect

Type: Single Module

Infill material and colour: Select or mix and match from the following options:

Fluted Rubber – Black (for interior and exterior use)

Ultragrip safety rubber – black (for sure footing in multi-directional entrances, interior and exterior use)

4 row bristle – black (for interior and exterior use)

Coral Classic – 4701 anthracite/4730 raven black/4727 navy blue/4764 taupe/4744 espresso/4750 warm black – for interior use only

Thickness:	10mm//17mm/22mm [Select required option]

Size: As drawing ref.

Matwell Frame: Select from the following options:

Existing

Recessed Aluminium AMF253/AMF346/AMF506

Ramped Aluminium: 12mm ARF 50 / 17mm ARF 70

[Note that 22mm connect is not designed for surface mounting]

Installation: Forbo Flooring recommends that Nuway Entrance systems are installed by a member of the FESSI (Forbo Entrance Systems Specialist Installer) network, details of which can found at the following link www.forbo-flooring.co.uk/FESSI

Construction options (all dimensions are approximate):